Instruction for Capillaries ROI Analysis ImageJ Macro

Purpose:
This macro processes the Z-stacked image in ImageJ, extracting intensity profiles from "Green" and "Red" channels across all drawn regions of interest (ROIs). It saves the extracted data as a CSV file ("NAME_ME_!!.csv"), including distance values for "Green" intensity and corresponding "Red" intensity values. Due to ImageJ's formatting limitations, the CSV file combines all ROI data longitudinally (in a single set) rather than side by side.

Running the Macro:

- Set File Paths:

  - Update the Z_Stack_Image_name variable to the filename of the image you wish to analyze.

  - Set the desktopPath to the location where you want the CSV file to be saved.

  - Modify the csvFilePath to your desired filename for the output.

- Ensure Images are Open:

  - After adjusting intensity and split channel (refer to ImageJ instruction by Medha). Ensure the "Green" and "Red" channel images in ImageJ is open before running the macro.

  - The Red and Green images should be named automatically with the format "C1-" and "C2

- Execute the Macro: At the top of the ImageJ tool bar, click “Plugin” |> “Macro” |> “Edit”. Which should allow you to both run and edit the macro

Editing the Macro:

- If you need to change the way ROIs are processed or the data saved, adjust the processChannel() function. For example, you can add new measurements or change how results are saved to the CSV.

- Edit with ChatGPT: Copy the code and specify any changes you need, such as image paths, channel handling, or data processing adjustments.

Instruction for "CSV Processing and Compilation" Python Script and Bash Shell

Purpose:
After running the previous ImageJ analysis, this Python script processes the CSV files in a specified folder, extracting and rearranging data according to the 'ROI' index. It saves the combined data into an Excel file ("Combined & Rename.xlsx"), with each CSV file represented as a separate sheet.

Running the Script:

- Set Up Python Environment, ensure you have Python installed, along with the required packages (pandas and openpyxl).

- Copy the Python into a new folder.

- Edit the corresponding User and python file folder, in the batch file

- Left-click the batch file/bash shell, it should:

  - For each file, remove the first column and extract data grouped by the 'ROI index' column.

  - Renames the columns, adds an 'ROI' cell, and combines the results into a single Excel file.

  - The output Excel file is saved as "Combined & Rename.xlsx" in the same folder.

Editing the Script:

- Remember to modify the folder path in the command line of the bash shell to suit your directory structure.

- If you need to include different columns or adjust data processing logic, modify the column manipulation lines in the script.

- Copy the code and specify any changes needed, such as column adjustments, grouping logic, or output formatting.